SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

Dakshin S (192511431)

Code of Conduct:

I ______________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: DAKSHIN S

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network

infrastructure for its academic and administrative departments. The network

administrator has been allocated the IP address block 192.168.100.0/24. Based

on the expected number of devices, the following subnet masks have already

been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the

following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid host IP address range for each subnet.

3. Calculate the number of usable host addresses available in each subnet.

4. Determine the remaining unused IP address range within the given

network block.

Answer:

Subnet Allocation Overview

- Total IP Block: 192.168.100.0 /24 (Total addresses: 256, Range: .0 to .255)

- School of Computing: /25 (Subnet Mask: 255.255.255.128)

- School of Electronics: /26 (Subnet Mask: 255.255.255.192)

- School of Mechanical Engineering: /27 (Subnet Mask: 255.255.255.224)

1. Subnet Breakdown and Calculations

1. School of Computing (/25)

- Network Address: 192.168.100.0

- Subnet Mask: 255.255.255.128

- Total Addresses: 128

- Usable Host IP Range: 192.168.100.1 to 192.168.100.126

- Broadcast Address: 192.168.100.127

- Usable Host Addresses: 126

2. School of Electronics (/26)

(Starts immediately after the Computing subnet)

- Network Address: 192.168.100.128

- Subnet Mask: 255.255.255.192

- Total Addresses: 64

- Usable Host IP Range: 192.168.100.129 to 192.168.100.190

- Broadcast Address: 192.168.100.191

- Usable Host Addresses: 62

3. School of Mechanical Engineering (/27)

(Starts immediately after the Electronics subnet)

- Network Address: 192.168.100.192

- Subnet Mask: 255.255.255.224

- Total Addresses: 32

- Usable Host IP Range: 192.168.100.193 to 192.168.100.222

- Broadcast Address: 192.168.100.223

- Usable Host Addresses: 30

2. Remaining Unused IP Address Range

- Total allocated addresses so far: 128 (Computing)+64 (Electronics)+32 (Mechanical)=224 addresses.

- Last allocated IP: 192.168.100.223 (Broadcast address of the mechanical engineering subnet).

- Total IP space available: 256 addresses (.0 to .255).

- Remaining Unused Range: 192.168.100.224 through 192.168.100.255 (Total of 32 unassigned IP addresses).

2. A multinational software company is expanding its headquarters and has been

assigned the private IP block 10.10.0.0/16. Different departments require

different subnet sizes based on the number of employees. The network

administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid range of host IP addresses.

3. Calculate the number of usable hosts in each subnet.

4. Identify the unused IP address range remaining within the allocated IP

block.

Answer:

Subnet Allocation Overview

- Total IP Block: 10.10.0.0 /16 (Total addresses: 65,536, Range: 10.10.0.0 to 10.10.255.255)

- Software Development: /24 (Subnet Mask: 255.255.255.0)

- Quality Assurance: /25 (Subnet Mask: 255.255.255.128)

- Human Resources: /26 (Subnet Mask: 255.255.255.192)

- Executive Management: /27 (Subnet Mask: 255.255.255.224)

1. Subnet Breakdown and Calculations

1. Software Development (/24)

- Network Address: 10.10.0.0

- Subnet Mask: 255.255.255.0

- Total Addresses: 256

- Usable Host IP Range: 10.10.0.1 to 10.10.0.254

- Broadcast Address: 10.10.0.255

- Usable Host Addresses: 254

2. Quality Assurance (/25)

(Starts immediately after the Software Development subnet)

- Network Address: 10.10.1.0

- Subnet Mask: 255.255.255.128

- Total Addresses: 128

- Usable Host IP Range: 10.10.1.1 to 10.10.1.126

- Broadcast Address: 10.10.1.127

- Usable Host Addresses: 126

3. Human Resources (/26)

(Starts immediately after the Quality Assurance subnet)

- Network Address: 10.10.1.128

- Subnet Mask: 255.255.255.192

- Total Addresses: 64

- Usable Host IP Range: 10.10.1.129 to 10.10.1.190

- Broadcast Address: 10.10.1.191

- Usable Host Addresses: 62

4. Executive Management (/27)

(Starts immediately after the Human Resources subnet)

- Network Address: 10.10.1.192

- Subnet Mask: 255.255.255.224

- Total Addresses: 32

- Usable Host IP Range: 10.10.1.193 to 10.10.1.222

- Broadcast Address: 10.10.1.223

- Usable Host Addresses: 30

2. Remaining Unused IP Address Range

- Total allocated addresses so far: 256+128+64+32=480addresses (spanning from 10.10.0.0 to 10.10.1.223).

- Total IP space available in /16: 65,536 addresses (10.10.0.0 to 10.10.255.255).

- Remaining Unused Range: 10.10.1.224 through 10.10.255.255 (Total of 65,056 unassigned IP addresses).

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to

support thousands of connected medical devices and IoT sensors. The hospital

has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

1. Convert the given IPv6 address into its compressed notation.

2. Expand the compressed address back into its full hexadecimal

representation.

3. Identify the network prefix using the /64 prefix length.

4. Calculate the total number of host addresses supported within the

subnet.

5. the total number of possible host addresses available within this network.

Answer:

1. Compressed Notation

To compress an IPv6 address, you omit leading zeros within any 16-bit block (hextet) and replace the longest consecutive sequence of all-zero hextets with a double colon (::).

- Given Address: 2001:0db8:abcd:0000:0000:0000:0000:0000

- Compressed Form: 2001:db8:abcd::

2. Full Hexadecimal Representation

To fully expand an IPv6 address, ensure every single one of the eight hextets contains exactly four hexadecimal digits by padding any missing values with leading zeros.

- Full Representation: 2001:0db8:abcd:0000:0000:0000:0000:0000

3. Network Prefix (using /64 prefix length)

In IPv6, a standard subnet for a local network or VLAN is a /64. This means the first 64 bits are dedicated to the network routing prefix, and the remaining 64 bits are reserved for the interface identifier (hosts).

- Network Prefix: 2001:0db8:abcd:0000::/64 (or simply 2001:db8:abcd::/64)

4. Total Number of Host Addresses Supported within the Subnet

Unlike IPv4, IPv6 does not reserve separate addresses for a network ID or a broadcast address (broadcast traffic is replaced by multicast in IPv6). Every single address in the host portion is available for use by an interface.

- Host Bits: 128 total bits-64 network bits=64 bits

- Total Host Addresses: 264

- Value: 18,446,744,073,709,551,616 addresses (approx. 18.4 quintillion addresses)

5. Total Number of Possible Host Addresses Available within this Network

Since the assigned block itself is a single /64 subnet, the total number of host addresses available in this network is identical to the calculation above:

- Total Host Addresses: 264=18,446,744,073,709,551,616

4. An Internet Service Provider (ISP) connects multiple customer networks

through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

1. The destination subnet to which the packet belongs.

2. The matching routing table entry.

3. The next-hop network/interface used to forward the packet.

4. The default route that should be used if the destination does not match

any routing table entry.

ANSWER:

1. Destination Subnet to Which the Packet Belongs

- Destination IP: 192.168.2.45

- Matching Subnet: 192.168.2.0/24

- Explanation: A /24 subnet mask means the first three octets (192.168.2) represent the network portion. Since the packet's IP has 192.168.2 in the first three octets, it belongs to the second subnet in the routing table.

2. Matching Routing Table Entry

- Matching Entry: 192.168.2.0/24

- Explanation: Using Longest Prefix Matching, the router evaluates the table against the destination IP and finds an exact match for the /24 network prefix belonging to 192.168.2.0/24.

3. Next-Hop Network/Interface Used to Forward the Packet

- Next-Hop / Interface: Directly Connected (or the specific local interface associated with the 192.168.2.0/24 network on the core router).

- Explanation: Because the destination IP falls within a subnet directly connected to the router's routing table, the router does not need to forward it to another router; it delivers the packet directly to the local segment via the corresponding interface (e.g., GigabitEthernet0/1 or similar local port).

4. Default Route That Should Be Used If the Destination Does Not Match Any Routing Table Entry

- Default Route: 0.0.0.0/0

- Explanation: In IPv4 routing, a default route uses the prefix 0.0.0.0/0 (often referred to as the "quad-zero" route). Any packet whose destination IP does not match any specific entry in the routing table is forwarded to the gateway specified by this default route.

5. A company establishes two office departments connected through a Layer-3

router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each LAN.

2. Identify the valid host IP address range.

3. Suggest suitable IP addresses for three computers in each LAN.

4. Identify the default gateway that hosts in each LAN should use for

communication.

ANSWER:

1. Network Address and Broadcast Address

Administration LAN (192.168.10.0/26)

- Network Address: 192.168.10.0

- Broadcast Address: 192.168.10.63

Finance LAN (192.168.10.64/26)

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

2. Valid Host IP Address Range

- Administration LAN (/26): 192.168.10.1 to 192.168.10.62

- Finance LAN (/26): 192.168.10.65 to 192.168.10.126

3. Suitable IP Addresses for Three Computers in Each LAN

(Note: The gateway addresses are excluded to prevent IP conflicts with the router).

Administration LAN Computers:

- Computer 1: 192.168.10.2 (Subnet Mask: 255.255.255.192)

- Computer 2: 192.168.10.3 (Subnet Mask: 255.255.255.192)

- Computer 3: 192.168.10.4 (Subnet Mask: 255.255.255.192)

Finance LAN Computers:

- Computer 1: 192.168.10.66 (Subnet Mask: 255.255.255.192)

- Computer 2: 192.168.10.67 (Subnet Mask: 255.255.255.192)

- Computer 3: 192.168.10.68 (Subnet Mask: 255.255.255.192)

4. Default Gateway for Hosts in Each LAN

- Administration LAN Gateway: 192.168.10.1

- Finance LAN Gateway: 192.168.10.65