Dakshin S

Dakshin S(192511431)

Question 1: Missing layers in the concept map

Application

Presentation

Session (missing)

Dialogue control & sync

Transport

Network

Data link (missing)

Framing, MAC, error check

Physical

Theory

The two missing layers are Session and Data Link.

Session layer establishes, manages, and terminates the dialogue between two communicating 
applications. It handles synchronization (checkpoints so a long transfer can resume rather than restart) 
and dialogue control (coordinating who transmits when), keeping the conversation coherent across 
multiple exchanges.

Data Link layer sits between Network and Physical. It organizes raw bits into frames, performs error 
detection (CRC/checksums) on a hop-by-hop basis, handles MAC addressing so frames reach the right 
device on the local segment, and manages flow control between directly connected nodes.

Without Session, applications would lose conversational structure across exchanges. Without Data Link, raw 
bits would have no framing, local addressing, or error checking — together the two layers bridge application 
logic and physical transmission.

Question 2: Data units and their OSI layers

Application / Presentation / Session

produces Data

Transport layer

produces Segment

Network layer

produces Packet

Data link layer

produces Frame

Physical layer

produces Bits

Theory

Each layer wraps (“encapsulates”) the layer above's output with its own header as data moves down the 
stack.

Data (the application message) is handed to Transport, which breaks it into Segments, adding port 
numbers and sequencing info — enabling reliable, ordered delivery and multiplexing between applications.

Segments become Packets at the Network layer, wrapped with source/destination IP addresses — 
enabling routing across different networks.

Packets become Frames at the Data Link layer, wrapped with MAC addresses and a trailer for error 
checking — enabling delivery to the correct device on the local link.

Frames become Bits at the Physical layer — encoded as electrical, optical, or radio signals for actual 
transmission. This layered encapsulation (and reverse decapsulation at the receiver) is what makes 
large-scale, heterogeneous networks manageable and reliable.

Question 3: Local delivery vs. global delivery

| Data link layer | MAC address | Local delivery |
| --- | --- | --- |

| Network layer | IP address | Global delivery |
| --- | --- | --- |

Theory

MAC addresses are physical/hardware addresses, meaningful only within a single local network segment 
(LAN). Switches use them to deliver frames to the correct device on that segment.

IP addresses are logical addresses identifying a device's network and host location globally, across 
interconnected networks. Routers use them to forward packets network to network until they reach the 
destination.

Global delivery cannot happen without local delivery at each hop. IP decides the overall path; at every hop 
(source → router → router →destination), the Data Link layer re-wraps the packet in a frame with the 
correct local MAC address for that segment. This repeated handoff at each router is what lets data travel 
reliably across the entire internet.

Question 4: Error control across layers

Error control

Data link layer 	Transport layer

Error detection 	Error correction

Theory

Data Link (Error Detection) : uses CRC/checksums to detect corruption over a single physical link. It 
typically only detects errors and discards the bad frame — it does not guarantee end-to-end retransmission.

Transport (Error Correction/Recovery) : uses checksums, acknowledgments, sequence numbers, and 
retransmission (e.g. TCP) to ensure the complete end-to-end message eventually arrives correct, even if a 
frame was lost or corrupted somewhere along the path.

This is defense-in-depth: Data Link catches problems early, close to where they occur, preventing bad data 
from being forwarded further. Transport compensates by working end-to-end, since Data Link alone cannot 
guarantee the message survives multiple hops intact. Together they balance efficiency with reliability.

Question 5: Web browsing across OSI layers

Application

HTTP request sent

Presentation

TLS encryption applied

Session

Connection held with server

Transport

TCP handshake, segments

Network

IP routing across networks

Data link

Framing per hop, MAC address

Physical

Bits over cable or Wi-Fi

Theory

Each layer plays a specific role in loading a page: Application sends the HTTP request, Presentation applies 
TLS encryption, Session holds the connection, Transport handles the TCP handshake and segmentation, 
Network routes packets via IP, Data Link frames and addresses each hop, and Physical carries the bits.

Because OSI layers are sequentially dependent , a failure at any single layer cascades upward and breaks 
the whole process — e.g. a TLS error at Presentation blocks the page even if Physical and Data Link work 
perfectly.

This is why network troubleshooting conventionally works bottom-up : check physical connectivity first, then 
move up layer by layer.

Question 6: Troubleshooting a network fault

Physical

Working

Data link

Working

Network

Failed — fault originates here

Transport

Failed — consequence of Network

Application

Failed — consequence of Network

Theory

Physical and Data Link working means: the cable/Wi-Fi signal is present, and local communication with the 
immediate neighbor (switch/access point) via MAC addressing is functional.

The failure starts at Network, and Transport/Application fail only as a consequence — they depend on 
Network to function, not because they have independent problems.

Likely causes: incorrect or missing IP configuration, wrong gateway or subnet mask, no route to the 
destination network, or a down router.

Troubleshooting value : the map isolates the fault immediately (no need to check cables or MAC-level 
issues), explains the cascading failures as expected rather than separate bugs, and directs corrective action 
precisely to Network-layer fixes — turning a vague “the internet isn't working” complaint into a precise, 
layer-specific diagnosis.